- Develop smart noise sensors equipped with data analytics capabilities that can be deployed across a city.

- Build online platforms or mobile apps that allow residents to report noise disturbances.

- Combine this crowdsourced data with sensor data for a comprehensive understanding of noise issues and their spatial distribution.

Create dynamic noise maps that update in real-time or on a frequent basis, providing policymakers and residents with up-to-date information about noise levels and sources.

s.

Design principle:

CODE FOR NOISE POLLUTION MONITORING:

Import machine

import time

import urequests

import ujson

import network

import math

# Define your Wi-Fi credentials

wifi_ssid = 'Wokwi-GUEST'

wifi_password = ''  # Replace with the actual Wi-Fi password

# Connect to Wi-Fi

wifi = network.WLAN(network.STA_IF)

wifi.active(True)

wifi.connect(wifi_ssid, wifi_password)

# Wait for Wi-Fi connection

while not wifi.isconnected():

pass

# Define ultrasonic sensor pins (Trig and Echo pins)

ultrasonic_trig = machine.Pin(15, machine.Pin.OUT)

ultrasonic_echo = machine.Pin(4, machine.Pin.IN)

# Define microphone pin

microphone = machine.ADC(2)

calibration_constant = 2.0

noise_threshold = 60  # Set your desired noise threshold in dB

# Firebase Realtime Database URL and secret

firebase_url = 'https://noise-pollution-bd0ab-default-rtdb.asia-southeast1.firebasedatabase.app/'  # Replace with your Firebase URL

firebase_secret = 'nBsgyQFTqHUe4qExlaZX6VL3mpf5gn6BlpnMiuR0'  # Replace with your Firebase secret

def measure_distance():

# Trigger the ultrasonic sensor

ultrasonic_trig.value(1)

time.sleep_us(10)

ultrasonic_trig.value(0)

# Measure the pulse width of the echo signal

pulse_time = machine.time_pulse_us(ultrasonic_echo, 1, 30000)

# Calculate distance in centimeters

distance_cm = (pulse_time / 2) / 29.1

return distance_cm

def measure_noise_level():

# Read analog value from the microphone

noise_level = microphone.read()

noise_level_db = 20 * math.log10(noise_level / calibration_constant)

return noise_level, noise_level_db

# Function to send data to Firebase

def send_data_to_firebase(distance, noise_level_db):

data = {

"Distance": distance,

"NoiseLevelDB": noise_level_db

}

url = f'{firebase_url}/sensor_data.json?auth={firebase_secret}'

try:

response = urequests.patch(url, json=data)  # Use 'patch' instead of 'put'

if response.status_code == 200:

print("Data sent to Firebase")

else:

print(f"Failed to send data to Firebase. Status code: {response.status_code}")

except Exception as e:

print(f"Error sending data to Firebase: {str(e)}")

try:

while True:

distance = measure_distance()

noise_level, noise_level_db = measure_noise_level()

print("Distance: {} cm, Noise Level: {:.2f} dB".format(distance, noise_level_db))

if noise_level_db > noise_threshold:

print("Warning: Noise pollution exceeds threshold!")

# Send data to Firebase

send_data_to_firebase(distance, noise_level_db)

time.sleep(1)  # Adjust the sleep duration as needed

except KeyboardInterrupt:

print("Monitoring stopped")